DIMENSIONAL ANALYSIS RELATIVE

TO MULTICOMPONENT MIXING AND SEPARATION PROCESSES

Vello Press, Lubomira Broniarz - Press, Jan Szymanowski

Poznan University of Technology, Institute of Chemical Engineering,

PL 60-965 Poznan, pl. M. Sklodowskiej-Curie 2, Poland

A common approach to mass transfer processes in multicomponent and multiphase media from simplified point of view of mechanics is put forward. On the basis of dimensional analysis dimensionless criterial parameters are determined which enable to provide quantitative characteristics of media both in local and integral scale. The method is based on the distinction of n-1 independent variables (so called mass moduli MM) in n-component media as mass fraction ratios of arbitrary component (j) to a selected basic one (i):

GjGi=gjgi=Mjim 						(1)

The redistributions of mass fractions of individual components are introduced by differences in current and initial values of MM. To provide common contributions for both inert and chemically reacting media the Schwab - Zeldovich binomials are made use of. Substantial changes in media are displayed by deviation in Schwab - Zeldovich conserved parameters as the functions of differences in values of MM1-2

βji=ai⋅Mi(i+j)m+aj⋅Mj(i+j)m 					(2)

Dβji=aj−ai⋅DMj(i+j)m 					(3)

As the criteria on completeness of mass transport process for n-component media n-1 dimensionless simplexes are derived as the degree of maximal possible deviation of Schwab - Zeldovich parameters of mutually based components

Kji=DβjiDβjimax 							(40

where Kji indicates the extent of changes (similarity of mechanical „saturation”) having occurred in multicomponent media, while a special relation of Kji called the conversion function

YKji=Kji1−Kji 						(5)

is proportional to mass ratio of a medium transported to a primary mass of multicomponent system.

When applying the dimensional analysis in a case of negative mass increments (separation processes) it is noticeable that turning about to opposite the signs of Dbji  the product of Kji and Y(Kji) remains invariable. So introducing this product as

Iji=Kji21−Kji							 (6)

a certain invariant of invertibility of mass transfer processes can be termed. The quantity of Iji is due to characterize periodically or randomly recurring processes. In particular, Iji can be derived for the case of mass concentration fluctuations of individual components in multicomponent media. The method under consideration enables to solve the following problems: calculating the bulk mass in a downstream contour of reactive mixing shear flow; calculating the mass of separated fractions from multicomponent bulk flow; determining a local degree of mixing and its relation to reaction progress variable in reactor volume if simple chemical reaction takes place; determining an invariant to characterize the mass concentration fluctuations in turbulent multicomponent media. The nonlinear relation between Kji and Y(Kji) and the presence of mass density fluctuations make possible to investigate increased mixing properties in turbulent media in correlation with invariant Iji.

If the current values of gj will vary between gjI and gjII(criterion (Kji)S between KjiI and KjiII, respectively) in volume element of turbulent medium, then invariant Iji can be modified to following relation

Iji'=KjiII−KjiI21−KjiI⋅1−KjiII					 (7)

Thus Iji’ is responsible to such a case of process when alternatively the direct and inverted changes of mass fraction from gjI to gjII take place. This pattern of phenomenon, if carried over to fluctuations in mass density, will give an opportunity for quantitative evaluation of these fluctuations of mass concentration in turbulent core of medium3.

Below the solution of two exemplary problems from chemical and power engineering are presented to illustrate some applications of the method under consideration. The preliminary assumptions on subject are taken as follows: reacting media are admitted to be continuous one of fluid or pulverized dispersion of solid in fluid; the summary effect of a chemical reaction can be modelled by the run of simple reaction of the type A + B ® C (simple chemically reacting system SCRS); the effective mechanism of a process is regarded as the confluence and mixing of reactive media jointly with a simultaneous finite rate chemical reaction.

Problem I. To determine a local degree of mixing (mixedness) c in the volume of flow reactor while both the mixing and the simple reaction A + B ® C are simultaneously to take place at finite rates. The solution of the problem is unfeasible in conventional way as in the case of mixing of nonreacting components of A and B by determination the local mass fraction of incoming component B:

χ=gBgBid							(8)

On the basis of dimensional analysis method a degree of mixing can be expressed as a ratio of criteria

χ=KBAKBAid 							(9)

The rate of c between the states of complete segregation and ideal mixing of components A and B is obtained on the basis of differences in reactive scalar parameters bBA in conformity to pattern when component B penetrates into the mass of component A. The effective formula is given by

χ=ηη+ΔβBA							(10)

where h is a reaction progress variable (reactedness) and

ΔβBA=gBidMBAmreal−gAid⋅η					(11)

MBAmreal is to be determined as the actual ratio concerning all elemental fractions of A and B in reactive medium.

Problem II. To determine the mass of entrained medium in downstream contour  of burning jet when the simple reaction of combustion A + B ® C takes place at finite rate in the flame. Conventional way for solution consists in measurements of velocity field in separate cross sections of jet flow jointly with local specification of fluid density and successive summation of local mass fluxes over the relevant cross section of the flow. Using the dimensional analysis method the measurement either of the flow velocity or its temperature is not obligatory within downstream contour of the flame. It is sufficient to fix the distribution of substantial parameters such as mass fractions of components in the volume of the flame with a view to provide the set of values of the function ΨKBA in volume elements in that volume. Then, on the basis of averaged over separate cross sections values of ΨKji after integration along the axis of the flame, the total excessive mass of entrained medium can be found by following governing equations (the concept of stream tube with permeable walls):

MΣ'Σm=0xqFm⋅FΨKBA⋅dF⋅dx				(12)

qFm=q0Fm⋅F0F							(13)

where q0F denotes initial mass flux averaged over initial cross section of the flow, F0 and F are equal to initial and current areas of cross sections, respectively.

The method was approbated by the calculation of the mass of recirculating gases and entrained excess air, as well as the mass of gasified fuel in the confined pulverized oil shale flame in an inverted combustion chamber4.

# REFERENCES

- Williams F.A., Combustion Theory, Reading Mass, Palo Alto, London, 1965.

- Bilger R.W., Turbulent Flows with nonpremixed Reactants, in: Turbulent Reacting Flows (ed. By P.A. Libby and F.A. Williams), Topics in Applied Physics, Vol. 44, pp. 65-113, Springer Verlag, Berlin, 1980.

- Press V., Broniarz-Press L., Szymanowski J.,  Dimensional Analysis Application to Multicomponent Mixing and Separation Processes, Two-Phase Flow Modelling and Experimentation, Vol. I, pp. 659-663, Edizioni ETS, Pisa, 1999.

- Press V., Szymanowski J., Dimensional Analysis Relative to Multicomponent Fluid Mixing, Proceedings of the 12 International Congress of Chemical and Process Engineering CHISA'96, Ref. No. 0500, pp. 1-10, Praha, 1996.