THE ROLE OF FINITE SPEED OF HEAT CONDUCTION IN TRANSIENT CONVECTIVE HEAT TRANSFER

H. Herwig

Technische Thermodynamik, TU Hamburg-Harburg

D-21073 Hamburg, Germany

In our study we address the question under which circumstances convective heat transfer modelling can be based on Fourier's "law" of heat conduction. In this law (which actually is a first but very good guess for a constitutive equation in the thermal energy equation) it is assumed that heat travels with infinite speed (ch=∞). Since, however, in real life chmust be finite (though it may be very large) the question arises for which cases a more sophisticated model is needed that may account for effects related to a finite value of ch.

.

From order of magnitude considerations it is easily deduced that for the Fourier law to be appropriate a time constant of heat conduction, τh, must be considerably smaller than that of the transient flow, τf. Here τf can be approximated by τf = LCUC with LC as a characteristic length and UCa characteristic velocity of the flow.

The question therefore is: Will the necessary condition

τh << LCUC

ever be violated? This will be discussed on the background of available data for τh and the controversial discussion whether τh can take large values for certain special materials.

References

Herwig, H.; Beckert, K. (2000): Fourier versus Non-Fourier Heat Conduction in Materials with a Non-homogeneous Inner Structure, Journal of Heat Transfer, Vol. 122, 363 - 365