UNSTEADY NATURAL CONVECTION HEAT TRANSFER IN A DOMAIN PARTLY FILLED WITH POROUS SUBSTRATE USING THE BRINKMAN-EXTENDED DARCY MODEL.

Dr. A. F. KHADRAWI

Mechanical Engineering, Al-Balqa’ Applied University

Al-Salt, Jordan

The problem of transient free convection in a domain partly filled with porous substrate is investigated using the Laplace transformation technique. A porous substrate attached to one of the heated walls is considered. The Brinkman-extended Darcy model is adopted to describe the hydrodynamics behavior of the porous domain. The effect of some parameters, such as thermal conductivity ratios and thermal diffusivity ratios, on the temperature and velocity distributions is investigated.

#### Introduction

The use of porous substrates to improve convective heat transfer in channels has many practical, geophysical, environmental, and technological applications. Examples include electronic cooling, chemical and nuclear reactors, heat transfer from hair covered skin, porous flat plate collectors, grain and food storage and drying, solidification of concentrated alloys, packed bed thermal storage, and fibrous and granular insulation where the insulation occupies only part of the space separating the heated and cooled walls. The present work considers the transient free convection fluid flow problem in a domain partially filled with porous substrates.

## Mathematical formulation

Consider an unsteady laminar fully developed free convection flow in a domain partly filled with porous material. The fluid is assumed to be Newtonian with uniform properties and the porous medium is isotropic and homogeneous. Also, it is assumed that both viscous dissipation and internal heat generation are absent. Using dimensionless parameters, the equations of motion and energy in both clear and porous domains are given as, respectively,

∂U1∂τ=∂2U1∂ Y2+θ1,		∂θ1∂τ= 1Pr ∂2θ1∂ Y2						                                    (1) Ca∂U2∂τ= μR∂2U2∂ Y2 − 1 Da U2+θ2  ,           ∂θ2∂τ= αRPr ∂2θ2∂ Y2                        (2)           with the following initial and boundary conditions:

U10,Y = U20,Y=0.0,           θ10,Y = θ20,Y=0.0

U1 τ, 0 =0.0 ,		θ1 τ, 0  =1.0	,     U2 τ, 1 =0.0	,	  θ2 τ, 1  =1.0

U1 τ, r = U2 τ, r,                  θ1 τ, r = θ2 τ, r                                              (3)

∂U1∂ Y τ, r = μR ∂U2∂ Y τ, r,            ∂θ1∂ Y τ, r = KR ∂θ2∂ Y τ, r

where the subscript 1 and 2 refer to the clear and porous domains respectively. In Eq. (2), Cais an acceleration coefficient tensor, and it depends on the geometry of the porous medium. The other parameters appearing in Eqs. (1-3) are defined as follows:

μR=μ2μ1,    KR=k2k1 ,      Da=KL22  ,        Pr=ν1α1,     r=L2L1,

Equations (1-2) are solved using Laplace transformation technique. Now with the notation that LUτ,Y=WS,Y and Lθτ,Y=VS,Y, these equations assume the following solutions

W1S,Y=C1eS Y+C2 e−S Y−C3Pr−1SeM1Y−C4Pr−1S e−M1Y                                 (4)

V1S,Y=C3eM1Y+C4 e−M1Y   			                                                                      (5)                         W2S,Y=C5eA Y+C6 e−A Y−C7μRPr .S/αR−AeM2Y−C8μRPr .S/αR−A e−M2Y   (6)

V2S,Y=C7eM2Y+C8 e−M2Y                                                                                      (7)

where M1=Pr S,       M2=Pr SαR,      A=CaμR S+1μR Da

Also, the Laplace transformation of the boundary conditions, yields:

W1S,0=0.0, V1S,0=1S, W2S,1=0.0, V2S,1=1SW1S,r= W2S,r  ,               V1S,r= V2S,r  ,∂W1∂YS,r=μR∂W2∂YS,r=0.0,  ∂V1∂YS,r=KR∂V2∂YS,r=0.0

The constant C1to C8are found after inserting Eqs. (4-7) into the boundary conditions. Equations (4-7) are inverted using a computer program based on the Riemann-sum approximation.

Results and discussion

Figure 2 shows the spatial temperature distribution at different thermal diffusivity ratios αR. Here, the porous domain is in direct contact with the heated wall. Porous domains of higher thermal diffusivity produce higher αR ratios. In this situation the porous layer diffuses more heat as its diffusivity increases and this leads to an increase in both domain temperatures. This is traced from another point of view by plotting the transient temperature distribution as shown in Fig. 3. Figure 4 shows the spatial velocity distribution at different thermal diffusivity ratios. Due to its additive microscopic frictional drag term (Darcy term), the porous domain has a much lower velocity as compared to the clear domain velocity. Porous domains with larger thermal diffusivity (or high αR) diffuse more heat from the wall to both domains and this in turn increases the buoyancy effect, which is the driving force for the flow. This leads to an increase in the velocity. Figure 5 shows the spatial temperature distribution at different thermal conductivity ratios KR. Here the thermal conductivity ratio KR may be increased by decreasing the clear domain thermal conductivity. This in turns leads to a reduction in the heat carried from the heated wall to both domains and as a result, lower temperatures are attained in this situation.

z

(1)             (2)

Clear          Porous                     y

L1

L2

Figure 1:  Schematic diagram of the problem

under consideration.

Figure 2: Spatial temperature distribution at different            Figure 3: Transient temperature distribution at

thermal diffusivity ratios.                                           different thermal diffusivity ratios

Figure(): Spatial velocity distribution at different               Figure(): Spatial temperature distribution at different

thermal diffusivity ratios.                                         thermal  conductivity ratios.