THE ORIGIN OF MULTIPLE SOLUTIONS IN UNSTEADY FREE

CONVECTION FLOWS IN POROUS MEDIA

E. MAGYARI1), I. POP2) and B. KELLER1)

1)Chair of Physics of Buildings, Institute of Building Technology

Swiss Federal Institute of Technology (ETH) Zürich, CH-8093 Zürich, Switzerland

2) Faculty of Mathematics, University of Cluj, R-3400 Cluj, CP 253, Romania

## A great deal of effort has been devoted during the past decades to understanding the convective heat transfer in a fluid-saturated porous medium subject to various additional effects depending on the applications in a variety of geophysical and technological problems. An exhaustive review of the topic of convective flow in porous media can be found in the books by Nield and Bejan (1999), Ingham and Pop (1998, 2002), Vafai (2000) and Pop and Ingham (2001).

## Similarity solutions of the boundary layer equations in fluid mechanics and heat transfer theory have proved to be very useful in the interpretation of certain flow and heat transfer characteristics. Although, exact solutions represent highly specialized situations, they do give clues concerning the nature of more realistic behaviour. Here we present new analytic solutions for two of the similarity cases identified by Johnson and Cheng (1978) for the unsteady free convection boundary layer flow over an impermeable vertical flat plate adjacent to a fluid saturated porous medium. These are the asymptotic solutions corresponding to an exponential  and a power law  variation of the surface temperature respectively, solutions which are valid at sufficiently large distances from the leading edge of the plate. It is shown that in the power law case, physical solutions only exist in the range  of the temperature exponent and they can be expressed in terms of Kummer’s confluent hypergeometric functions. For  exponentially decaying unique solutions were found, while in the range  both exponentially and algebraically decaying multiple solutions occur. The origin of the multiple solutions as well as the feasibility conditions of all the solutions mentioned above is discussed in detail.

Key words: unsteady free convection, vertical plate, porous medium, exact similarity solutions, multiple solutions, algebraic decay

### References

- D. B. Ingham and I. Pop (eds.), Transport Phenomena in Porous Media, Pergamon, Oxford (1998); vol.2 (2002).

- C. H. Johnson and P. Cheng, Possible similarity solutions for free convection boundary layers adjacent to flat plates in porous media, Int. J. Heat Mass Transfer, 21 (1978) 709-718

- D. A. Nield and A. Bejan, Convection in Porous Media (2nd edition), Springer, New York (1999).

- I. Pop and D. B. Ingham, Convective Heat Transfer: Mathematical and Computational Modelling of Viscous Fluids and Porous Media, Pergamon, Oxford (2001).

- K.Vafai (ed.), Handbook of Porous Media, Marcel Dekker, New York (2000).