TRANSIENT FLOW AND HEAT TRANSFER PHENOMENA IN INCLINED WAVY FILMS

- K.Serifi*, N.A.Malamataris†, and V.Bontozoglou*

*Department of Mechanical and Industrial Engineering,

University of Thessaly,

GR-38334 Volos, Greece

†Department of Mechanical Engineering

Technological Educational Institution of Kozani

Kozani, Greece

INTRODUCTION

Film flow occurs in a variety of process equipment, including condensers, falling film evaporators, absorption columns and two-phase flow reactors. In a range of Reynolds numbers of interest to such engineering applications (1<Re<400) the flat free surface is unstable and traveling waves develop. These surface waves have long been known to affect heat and mass transfer across the film1. However, the flow problem is extremely complex because the location of the free surface varies continuously and must be found as part of the solution. Consequently, efforts to explain the influence of waves on transport processes have been mostly heuristic2-5 and only few studies have adopted rigorous numerical solution of the flow and energy transport equations7-10.

Of particular interest among the various wave forms adopted by the free surface are solitary waves, ie. strongly nonlinear humps proceeded by capillary ripples, which attain a stationary shape and are separated by relatively long stretches of flat substrate. It has been observed6 that in many circumstances the downstream evolution of film flow tends to a series of solitary waves. This occurs directly when regular disturbances of low frequency are introduced at the inlet, and indirectly (through a sequence of nonlinear interactions) when the inlet disturbance consists of random noise.

Thus, the goal of the present contribution is to investigate computationally the dynamics of an unstable inclined free surface as well as its influence on heat transfer across the film. The full Navier-Stokes equation is solved by a finite-element method, delineating the evolution of the free surface and the flow field below the waves7. The energy equation is solved subsequently with the same method, providing information on the effect of waves on heat transfer. The flow is considered from the initial stages of linear instability up to the development of stationary traveling waves. Particular emphasis is placed on the role of solitary waves. The present study is closely related to recent work by Miyara10, who used a finite-difference method to investigate heat transfer on a vertical film.

MATHEMATICAL FORMULATION

Two-dimensional, gravity-driven flow along a constant-temperature plane with inclination φ is considered. A full description of the problem is provided by the continuity, Navier-Stokes and energy equations. Boundary conditions for the flow problem are no-slip and no-penetration along the wall y=0, and the kinematic condition and a balance of forces along the unknown free surface y=h(x,t). An additional boundary condition at the inlet

h(0,t)=H+Acos2πft		A<<H

introduces small periodic disturbances of frequency f, and an appropriate boundary condition is applied at the outlet to minimize upstream distortions. In the energy problem, the liquid enters with a uniform temperature T0, the wall is set at a constant temperature T1 and the air above the free surface is considered to provide a constant heat transfer coefficient. The complete flow and heat transfer problem is described by the Reynolds, Re, the Weber, We, and the Peclet, Pe, dimensionless numbers.

The unknowns are the velocities u and v, the pressure p, the temperature T and the location of the free surface. These are expanded in terms of Galerkin basis functions as follows:

u=i=19uiϕi,   v=i=19viϕi,   p=i=14piψi,   T=i=19Tiϕi,   h=i=13hiϕi

where ϕi are biquadratic and ψi bilinear basis functions. The governing equations and the kinematic boundary condition are weighted integrally with the basis functions and are evaluated numerically using nine-point Gaussian quadrature. Time integration is performed by a Crank-Nicolson scheme and the resulting system of nonlinear algebraic equations is solved with the Newton-Raphson method.

RESULTS AND DISCUSSION

The entire evolution process of inlet disturbances is followed computationally. Initial results of wave celerity and exponential growth rate are in agreement with the predictions of spatial linear stability analysis. The fully-developed wave profiles are also accurately captured, as demonstrated by comparisons with relevant data. The effect of disturbance frequency on the stationary form of the free surface is shown in Figure 1a-d.

Dimensionless downstream distance, x/hN

Figure 1: Spatial evolution of the free surface for a film with Re=19.33, We=5.43 and inclination φ=6.4º. The periodic inlet disturbances have frequency (a) 7 Hz, (b) 4.5 Hz, (c) 3 Hz and (d) 1.5 Hz.

All stages of downstream evolution are documented in detail. Frequency spectra of the temporal variation of the free surface at different locations along the wall indicate the transition from a monochromatic disturbance to the broadband Fourier structure of nonlinear waves.  Indicative profiles of the evolution towards a solitary hump are demonstrated in Figure 2.

The velocity field of solitary waves is found to exhibit a strong deviation from the parabolic shape in front of the main hump. A transient region of flow reversal is formed and travels with the wave celerity. This is shown to affect heat transfer from the wall in a fundamentally different way than the flow field produced by a series of closely-spaced periodic waves.

Figure 2: The nonlinear evolution towards a solitary wave in a flow with Re=19.33, We=5.43 and inclination φ=6.4º and inlet disturbances of frequency 1.5 Hz.

REFERENCES

1.  Dukler A. E., The role of waves in two-phase flow: some new understanding, 1976 Award Lecture Chem. Engng. Educ., , Vol. 11, pp 108-138, 1977.

2.  Brauner N. and Maron D. M., Characteristics of inclined thin films, waviness and the associated mass transfer, Intl. J. Heat Mass Transfer, , Vol. 25, pp 99-110, 1982.

3.  Jayanti S. and Hewitt G. F., Hydrodynamics and heat transfer of wavy thin film flow, Intl. J. Heat Mass Transfer,  Vol. 40, pp 179-190, 1997.

4.  Bontozoglou V., A numerical study of interfacial transport to a gas-sheared wavy liquid, Intl. J. Heat Mass Transfer,  Vol. 41, pp 2297-2305, 1998.

5.  Roberts R. M. and Chang H.-C., Wave-enhanced interfacial transfer, Chem. Eng. Sci.,  Vol. 55, pp 1127-1141, 2000.

6.  Liu J. and Gollub J. P., Solitary wave dynamics of film flows, Phys. Fluids, Vol. 6, pp 1702-1711, 1994.

7.  Malamataris N., Vlachogiannis M. and Bontozoglou V., Solitary waves on inclined films: Flow structure and binary interactions. Phys. Fluids, Vol. 14, pp 1082-1094, 2002.

8.  Ramaswamy B., Chippada S. and Joo S.W., A full-scale numerical study of interfacial instabilities in thin-film flows. J. Fluid Mech., Vol. 325, pp 163-194, 1996.

9.  Stuhltrager E., Miyara A. and Uehara H., Flow dynamics and heat transfer of a condensate film on a vertical wall-II. Flow dynamics and heat transfer. Int. J. Heat Mass Transfer, Vol. 38, pp 2715-2722, 1995.

10.  Miyara A., Numerical analysis of flow dynamics and heat transfer of falling liquid films with interfacial waves. Heat & Mass Transfer, Vol. 35, pp 298-306, 1999.