ONSET OF CONVECTIVE MOTION IN A HORIZONTAL FLUID LAYER HEATED FROM BELOW AND COOLED FROM ABOVE WITH CONSTANT HEAT FLUX

Min Chan Kim*, Lae Hyun Kim**, Chang Kyun Choi***
*Department of Chemical Engineering, Cheju National University,

Cheju 690-756, Korea

**Department of Chemical Engineering, Seoul National University of Technology,

Seoul 139-743, Korea

***School of Chemical Engineering, Seoul National University,

Seoul 151-744, Korea

A theoretical analysis of thermal instability driven by buoyancy forces in transient temperature fields is conducted in an initially quiescent, horizontal fluid layer heated from below and cooled from above with uniform heat flux. Under the principle of the exchange of stabilities, the stability analysis is performed on the basis of the propagation theory which adopts the thermal boundary-layer thickness as a characteristic length scaling factor and self-similar transformation. The prediction compares quite well with existing experimental result.

THEORETICAL ANALYSIS

When an initially quiescent fluid layer confined between two horizontal plates is heated rapidly from below and/or cooled rapidly from above, the basic temperature profiles of heat conduction develop with time and buoyancy-driven convection can set in at a certain time. In this transient system the critical time tc to mark the onset of convective motion becomes an important question. The related instability analysis has been conducted by using the frozen-time model1, propagation theory2, maximum-Rayleigh-number criterion3 and amplification theory4. The first two models are based on linear theory and yield the critical time as the parameter. The last model requires the initial conditions at the heating time t=0 and the criteria to define manifest convection.

Mathematical formulation

The problem considered here is a horizontal fluid layer confined between two rigid boundaries separated by a distance “d”, as shown in Fig. 1. The fluid layer is initially motionless at a constant temperature T0. For time t≥0, the fluid layer is heated from below and cooled form above with   constant heat flux qw. The schematic diagram of the basic system of pure conduction is shown in Fig. 1. The dimensionless basic temperature θ0 has the scale of qwdk, where k denotes the thermal conductivity. Under linear stability theory, the disturbances caused by the onset of thermal convection can be formulated, in dimensionless form, in terms of the temperature component θ1 and the vertical velocity component w1 as

1Pr∂∂τ−∇2∇2w1=−∇12θ1,                                                                                                                      (1)

∂θ1∂τ+Raqw1∂θ0∂z=∇2θ1,                                                                                                                          (2)

constant heat flux qw

d

z

rigid

g

rigid

constant heat flux qw

Fig. 1. Schematic diagram of system considered here.

where ∇2=∂2∂x2+∂2∂y2+∂2∂z2 and ∇12=∂2∂x2+∂2∂y2. The dimensionless velocity component has the scale of αd, the vertical distance d, and the temperature disturbance the scale of αν(gβd3), where α denotes the thermal diffusivity, ν the kinematic viscosity, g the gravity acceleration and β the thermal expansivity. Here Raqgβqwd4kαν is the Rayleigh number, τtαd2 the dimensionless time, and Prνα the Prandtl number. The proper boundary conditions are given by

w1=∂w1∂z=∂θ1∂z=0     at   z=0    and    z=1.                                                                            (3)

Our goal is to find the critical time τc to mark the onset of convective instability for a given Raq by using Eqs. (1)-(3). With the frozen-time model, the term involving ∂(⋅)∂τ is neglected and therefore the system becomes time-independent, the results are independent of Pr. With the amplification theory the proper initial conditions at τ=0 and the amplification factor to represent manifest convection are required. However, propagation theory is a rather simple, deterministic approach even though it involves the transient effect.

The present study will employ propagation theory for the stability analysis. Based on scaling analysis, the following amplitude relations can be obtained:

w1θ1~δT2~τ,  Raw1δT2Dθ0~θ1.                                                                                            (4a,b)

where Ra=RaΔT is the Rayleigh number based on the thermal boundary-layer thickness ΔT and the wall heat flux qw, and δTτ12 is the nondimensional thermal boundary-layer thickness. In propagation theory, RaRaqτ2 is assumed to be a constant. From the resulting perturbation equations the self-similar stability equations2 are obtained and the minimum value of Ra, i.e., Rac is obtained numerically.

Stability analysis results

The stability criteria for various Pr values are summarized in Fig. 2. It seems evident that the critical Rayleigh number Rac increases with a decrease in Pr and the Pr-effect becomes pronounced for Pr<1. For the high-Pr case, the long wave mode is preferred. This means that the inertia forces make the system more stable and the disturbance be confined within the narrow regions near the boundary.

Ward and Le Blanc5 measured the voltage variation with time in an electrochemical redox system where the Schmidt number Sc, which is equivalent to Pr in a heat transfer system, is larger than 1,000. For this electrochemical system the present τc-values are about one sixth of the experimental result τo, as shown in Fig. 3. Here τo represents the characteristic time to mark manifest convection. Foster6 commented that with correct dimensional relations τo≃4τc. This means that a fastest growing mode of instabilities, which set in at t=tc, will grow with time until manifest convection is detected at to≃4tc. A growth period will be required, as illustrated in Fig. 3. The validity of to≃4tc requires a further study but this relation is shown even in other transient diffusive systems2.

Fig. 2. Effect of Pr on critical condition.               Fig. 3. Comparison of theoretical results with

experimental one.

CONCLUSION

The onset of buoyancy-driven motion in a horizontal fluid layer heated from below and cooled from above with uniform heat flux has been analyzed analytically by using linear stability theory. The propagation theory and frozen-time model predict that the onset time of buoyancy driven motion is a function of the Rayleigh number and the Prandtl number. It seems that for high Pr case, the long wave mode is preferred and the inertia forces make the system more stable and the disturbance be confined within the narrow region. The present result shows that the propagation theory predicts the onset time quite well and the inertia term makes the system more stable.

This work was supported by a grant from the Hyocheon Academic Research Fund of the Cheju National University Development Foundation.

REFERENCES

- Morton, B. R., On the Equilibrium of a Stratified Layer of Fluid, J. Mech. Appl. Math., Vol. 10, pp 433-447, 1957.

- Kim, M. C., Park, H. K. and Choi, C. K., Stability of an Initially, Stably Stratified Fluid Subjected to a Step Change in Temperature, Teoret. Comput. Fluid Dynamics, Vol. 49, pp 49-57, 2002.

- Tan, K. K. and Thorpe, R. B., The Onset of Convection Caused by Buoyancy During Transient Heat Conduction in Deep Fluids, Chem. Eng. Sci., Vol. 51, pp 4127-4136, 1996.

- Foster, T. D., Stability of Homogeneous Fluid Cooled Uniformly from Above, Phys. Fluids, Vol. 8, pp 1249-1257, 1965.

- Ward, W. J. and Le Blanc, O. H., Rayleigh-Bénard Convection in an Electrochemical Redox Cell, Science, Vol. 225, pp 1471-1473, 1984.

- Foster, T. D., Onset of Manifest Convection in a Layer of Fluid with a Time-Dependent Temperature, Phys. Fluids, Vol. 12, pp 2482-2487, 1969.