Unsteady Flow Modelling in Turbine Stage

Prof. Francesco Martelli ,

Energetics Department, University of Florence Italy

The flow in the stage of any kind of turbomachinery is strongly unsteady for several reasons; the first and more relevant phenomena is related, of course,  to the relative motion of rotor and stator,  but other unsteady phenomena occur which can have  significant effect especially  on the   stage behaviour in off—design conditions , i.e. the   vortex shedding at trailing edge, the  tip leakage  vortex  which can be unstable  interacting with other unsteady and unstable situation in the flow passages. The effect of unsteadiness have significant effect on the performance of the stage, and actually it is demonstrated that the behaviour of a single row differs from  its  when located in a multiple row environments. In the turbine blades of the firsts stages of Gas Turbine , where metal temperature  are very important, the effect in not only on performance but on the blade heat transfer as well  and therefore on the possible life of the bucket  itself.  Further the unsteady forces can interact with blade elasticity  producing flow induced  vibration (flutter) with strong effect on structural survival of the  rows.

For long time the  stage have been computed, analysed and designed with steady assumption , mainly due to the complexity of unsteady flow, and   unsteady viscous effect modelling; more recently the improvements in computing capacity and the need to increase the efficiency in the last few  percentage points, the maximum   operating temperature and life of turbine stage has stressed this kind of modelling.

The  research activity  has been focused on three  different approach according to the target of the study ( unsteady aerodynamics, flutter,  unsteady losses et.):

- Fully unsteady calculation with different features  of the numerical solver (Rai, Giles, Martelli,…);

- Steady models based on the Adamczyk  averaging procedure ( (determinist stress);

- Quasi unsteady methods  where the unsteadyness is computed in  hybrid way ranging from linear decomposition  forms  to other numerical approaches based only on the unsteady  Boundary Conditions.

At present time  we are more interested in the study of turbine stage aerodynamics and heat transfer; these items require a fully unsteady simulation in order to catch  crucial points of the heat transfer phenomena and dissipation mechanism in the unsteady boundary layer and the losses related to the vortex shedding as well.

In order to match properly this kind of simulation two focal arguments exist, one is related to the physical modelling of turbulence and  transition, (the last especially  relevant in the heat transfer evaluation), and tough work is underway is this direction; the other is related to the numerical improvement in terms of both accuracy and storage & cpu time reduction, as the 3D unsteady calculations could require several millions  of mesh points and several thousands physical time steps  to get reasonable periodic configurations. In order to achieve these targets the solvers have to  use special treatment of boundary condition and to move towards massive parallel  computing architecture.

The paper will present the most up-to day results  in thes arear coming from the open literature and from the author research group  as well. Heron some example of preliminary two –dimensional  calculations , performed on a experimented turbine stage  is reported and the discrepancy between  steady solution solving the averaged equations and the mean solution averaging the unsteady solution from unsteady equations. In the next fig.1 typical mesh arrangement is reported.

Fig. 1

The effect of  steady/unsteady calculation is reported in Fig.2

STEADY SOLUTION          					 UNSTEADY SOLUTION

Fig. 2

The strong different behaviour of the two solutions can be realised looking at the fig.3 where steady flow picture is reported  against three configurations for  different time steps.

Steady Solution 					 Unsteady solution  time 1

Unsteady Solution time 2 				Unsteady Solution time 3

Fig. 3

Comments on the different numerical approaches, different  physical models and  sample of the flow phenomena comprehension  are widely addressed in the paper as well as the discussion of the future improvements under way in the research community and in the author research group.