HEAT TRANSFER CHARACTERISTICS UNDER ACOUSTIC CAVITATION

FIELD WITH ADDITIVE OF NANOMETER GRANULE

Dingwei Zhou **  Dengying Liu  Xuegong Hu

Institute of Engineering Thermophysics

Chinese Academy of Sciences, P.O.Box 2706, Beijing 100080, China

Abstract

Acoustic cavitation is a very complicated physical phenomenon generated by acoustic fields in liquid. Generally speaking, Cavitation bubbles collapse accompanied by several interesting phenomena, such as high temperature effect resulting in the formation of local hotspots in liquid, the interior temperature of which can be up to 6000K. Although plenty of work has been done to investigate the inherent mechanism of cavitation, the experimental data pertinent to heat transfer are scarce in open literatures1. Heat transfer enhancement by acoustic cavitation field is not the same as that by ultrasonic vibration in the liquid utilized in the mid-twenty centuries. The latter is a macroscopical means of heat transfer enhancement without cavitation bubbles, yet the former is to utilize thermal energy, which is released from collapsed microbubbles in the liquid activated by ultrasonic, to elevate heat transfer coefficient of the assembly. Single-phase convective heat transfer with acoustic cavitation is first examined by Bergles and Newell2 and followed by Wong and Chon3. The experimental results obtained by Zhou and Liu4 showed that the enhancement ratio can be up to 4 with a bundle of cavitation bubbles impinging dead against the heat transfer surface vertically, using ethanol as working fluid. For boiling heat transfer with acoustic cavitation field, it is confirmed by Zhou and Liu5 that boiling heat transfer can be enhanced by acoustic cavitation field due to the impingement and disturbance of cavitation bubbles on the test section surface, and to direct influence of cavitation bubbles on the nucleation, growth up and collapse of vapor embryo within cavities on the heat transfer surface, and to effective dissipation of latent heat of vapor with increasing nominal boiling areas.

On the other hand, recent research results revealed that the physical mechanisms of thermal transport in extremely small length and extremely short time could differ sensibly from those in the macro-scale world6. Nanometer material is a matter of which the size is bounded below nanometer magnitude. Due to the unusual characteristics of nanometer material held in the aspects of electricity, magnetism, light, heat and sensitivity, it has wide application foreground in the domain of pottery plasticizing, magnetism, electrons and optical material. Up to now, the number of papers, which have been published to introduce nanometer granule effect on heat transfer, is limited, especially on boiling heat transfer of horizontal circular copper tube under acoustic cavitation field.

The objective of this study is to investigate systematically the effects of nanometer granule on single-phase convective and boiling heat transfer of horizontal circular copper tube, with ethanol and acetone as working fluids. Included factors are cavitation intensity and orientation, fluid subcooling, nanometer granule concentration. In addition, the relation between vacuole radius and cavitation thresholds is analyzed theoretically to enucleate heat transfer mechanisms of nanometer granule under acoustic cavitation field.

The test chamber is a cubical stainless steel vessel with clear inside dimensions of 200mm×200mm×230mm. There are two viewing windows installed at the sides of the vessel for the purpose of observation and taking pictures of the test section. Cavitation bubbles generated by an ultrasonic vibrator, the intensity of which can be adjusted through ultrasonic generator. The test section assembly is a circular copper tube with outer and inner diameters of Ø20mm and Ø16mm respectively, horizontally fixed on the two sides of the chamber. All the thermocouples are connected to a model 7501 programmable scanner and the emf generated is measured by a model 7563 digital thermometer (6-1/2 digits). The uncertainty in heat transfer coefficients and heat flux did not exceed ±6% and ±5.5%, respectively.

Nanometer granule, of which the diameter is in the range of 80~100 nanometer, is made of calcium carbonate and dispersed uniformly under the impingement of cavitation bubbles after added to the working liquid. Under a certain conditions, single-phase natural convective heat transfer of ethanol with nanometer granule is decreased slightly. When cavitation bubbles act on the test section assembly, it seems that a decreasing trend becomes apparent with elevating cavitation intensity. The reason is that heat transferred by convective decreases with increasing movement of nanometer granule in liquid.

With increasing nanometer granule concentration, single-phase convective heat transfer remains unchanged, yet boiling heat transfer can be enhanced slightly. But the latter is still weakened by comparing with that without additive of nanometer material in liquid. With the variation of cavitation intensity and orientation, as shown in Fig.1 and Fig.2 respectively, boiling heat transfer of

| Fig.1 Cavitation intensity effect | Fig.2 Cavitaiton orientation effect |
| --- | --- |

horizontal circular copper tube can be enhanced distinctly. Without the action of cavitation bubbles, boiling hysteresis occurs with acetone as working fluid. However, it disappears with increasing cavitation bubbles due to the latter activating the vapor embryo within cavities on the heat transfer surface. Information on the influence of subcooling on nucleate boiling is relatively scant in open literature. Under acoustic cavitation field, fluid subcooling has an apparent influence on boiling heat transfer curves depicted in a form of q" ~ ΔTsat and h ~ q".

The theoretical results show that the diameter of nanometer granule is far below the cavities diameter on the heat transfer surface. Nanometer granule accumulates gradually first within cavities, and then on the heat transfer surface under the help of cavitation bubbles so that the surface roughness of the test section became more smooth. A decreasing number of activation nuclear is recorded and boiling heat transfer is therefore weakened.

Keywords: Acoustic Cavitation  Boiling Heat Transfer  Nanometer Granule  Hysteresis

### CONCLUSIONS

The most important findings of this study are:

1. Boiling heat transfer can be enhanced slightly with increasing nanometer granule concentration, but it is still weakened by comparing with that without additive of nanometer material in liquid.

2. The variation trend of boiling curve with cavitaiton orientation, intensity and fluid subcooling will not vary with the adding of nanometer granule in working fluid.

3. Boiling hysteresis phenomena occur with decreasing fluid subcooling under acoustic cavitation field, using acetone with nanometer granule as working fluid.

4. Nanometer granule has an important influence on boiling heat transfer of horizontal circular copper tube under acoustic cavitation field because the former makes the surface roughness of the test section assembly become smaller.

Acknowledgements: This project supported by the National Key Basic Research Science Foundation of China (Grant No.G2000026305).

## References

- Bergles, A.E., Handbook of Heat Transfer Fundamentals (ed. Rohsenow, W.M. et al.), McGraw-Hill, New York, 1998.

- Bergles, A.E. and Newell, P.H., The Influence of Ultrasonic Vibrations on Heat Transfer to Water Flowing in Annuli, Int. J. Heat Mass Transfer, Vol.8, No.6, pp1273~1280, 1965.

- Wong, S.W. and Chon, W.Y., Effect of Ultrasonic Vibrations on Heat Transfer to Liquids by Natural Convection and by Boiling, AIChE J, Vol.15, No.2, pp281~283, 1969.

- Zhou, D.W. and Liu, D.Y., Experimental Study on Heat Transfer Enhancement of Single Phase Convection under Acoustic Cavitation Field, Accepted by Progress in Natural Science (in Chinese, in press), 2001.

- Zhou, D.W. and Liu, D.Y, Boiling Heat Transfer with Acoustic Cavitation, Accepted by Progress in Natural Science, (in press), 2001.

- Guo, Z.Y., Size Effect on Flow and Heat Transfer Characteristics in MEMS, Proceedings of the International Conference Heat Transfer and Transport Phenomena in Microscale, Banff, Canada, October 15-20, 2000, pp 24-31.