DESigned distance functions FOR PIV problems

Sergey Butenkov* and Juan Jimenez**
*Laboratory of Mathematical Problems in Artificial Intelligence
Nekrasovsky Lane, 44, Taganrog, 347922, Russian Federation
**176 Old Graduate College
P. O. Box 1586, Princeton NJ 08544, USA

PIV provides the capability of obtaining spatial measurements versus single point measurements from hot wire anemometry or Pitot tube. PIV enhances the capability of seeing coherent structures in the flow, and provides detailed data that may be used for direct comparison with numerical simulations. We develop new mathematical results for particles tracking and measuring software in new experimental setup, which are assuming unknown correspondence between the particles sets.

## BACKGROUND

Well-known feature point tracking algorithms1,2,3 are not essentially differ from local scale-invariant features recognition algorithms4 . Very similar approaches for feature extraction3,4 and feature points tracing3,5 are used for PIV and recognition problems. We discuss two important parts of full PIV problem:

- selected feature points filtration;

- feature points (or points sets) tracking.

This subproblems are considered on the new mathematical basis6, related with model-based methods and implicit functions technique. New kinds of mathematical models for a wide range of feature points sets are presented.

## preprocessing for particles image

There are any ways to images features extraction2, but assume we have prepared for extraction grayscale image with many particles. Notice that consecutive images of original flows may have different average brightness. It is the lose conditions for tracking algorithms accomplishment. In this paper we can use the simplest brightness independent technique, related with binary features images:

bi,j∈0,1 i=1, ..., n; j=1, ..., m					(1)

We can process original grayscale image

gi,j∈0, ..., N i=1, ..., n; j=1, ..., m,					(2)

where N– maximal brightness value, by means of two levels segmentation α1 and α2:

bij=1, if (gij>α1)∧(gij<α2)0 otherwise.					(3)

The next possible way to segmentation improvement related with local segmentation levels αij1 and αij23. Processed binary image contains a lot of randomly placed points and points clusters on the plain. The main mathematical problem of features tracking is concluded in designing of distance functions5,9 for the arbitrary sets of points. Points tracking may be realized as point sets translations estimation for consecutive images. Assumed5 that points translations must be rigid (affine) for the fields of velocities estimation. Nevertheless, for the important problems of turbulent flows velocities estimation this supposition is not correct.

## new distance functions definitions

New analytical principles for distance functions design are presented in monograph6 and evolved in papers7,8 . Let Rn– algebraic space. Distance function dR:Rn×Rn→R for Rn usually defined by means of next axioms:

1. dR(x1,x2)≥0, dR(x1,x1)=0, dR(x1,x2)=0 ⇔x1=x2;

2. dR(x1,x1)=dR(x2,x1);								        (4)

3. dR(x1,x2)+dR(x1,x3)≥dR(x2,x3) ∀ x1,x2,x3∈Rn.

In particular, basic Hilbert distance function9 presented as:

dHm(x1,x2)=i=1nxi1−xi2m1m,						(5)

where m≥1. Pair of (Rn,dR) defines the metric space. Each space element x1,x2, ... ∈Rn usually be denominated as space point. Define point set object (PSO) in Rn with dimension n and size k as:

Wn=(x11, ..., xn1), ..., (x1k, ..., xnk).					(6)

For the flat images k- size PSO, related with (1) or (2) defined as W2=(x11,x21), ..., (x1k,x2k). For the tracing and classification problem solutions we needs distance functions for the point sets objects (6). Usually extreme distance functions are used for this purposes:

dW(W1n,W2n)=infx∈W1n, y∈W2ndR(x,y).				(7)

Very often similarity measures5 sW(W1n,W2n) for PSO (6) are used. Like measures are based on generalized objects descriptions, for example, on angles between object vectors in Rn, and so on. Assumed equal sizes and dimensions for W1n and W2n objects.

Define new conception for parameterize point set models7,8  by means of implicit function

ω(x,p)=0,							(8)

where x∈Rn, x=(x1, ..., xn) – model coordinates, p∈Rm p=(p1, ..., pm) – model parameters. Model function ω may be acquired by means of distance functions dR (4),(5). For the flat images we can define model function inR4 as:

ω(x1,x2,p1,p2)=dR(x1,x2,p1,p2)=(x1−p1)2+(x2−p2)2=0,		(9)

For the set of points and it’s models(9) we can use special algebraic operations6,10 named as R- operations for complex point set model design We needs R– disjuncitons  for the complex model function as implicit function Rn×Rm→R, where m– particular models ω(x,p) dimension:

ωWn(x,p)=i=1, ..., kωi(x,p).					(10)

Now we define object distance function as

dR(W1n,W2m)=1bi=1bωi(W1n,W2m),					(12)

where b=max(k,m). Notes, that W1n and W2mobjects must have equal dimensions n and they can have different sizes k and m (it is not vectors). For (12) we can prove next theorem:

Theorem: function (12) will provide main axioms (4) for objects W1n and W2m if particular models are presented as distance functions on Rn.

## CONCLUSION

Presented constructive approach to distance functions design was realized in software for PIV experimental setup and estimated as very robust and exact particles tracing method. Basic theoretical results may be used for non-flat problems, for example, for 3-D object images recognition8 .

## REFERENCES

- I. Grant Particle Image velocity: a review. Proc. Institution of Machanical Engineers, 211 Part C:55-76, 1997.

- J. Shi, C. Tomasi Good features to track. In Proc. IEEE Conf. On Computer Vision and Pattern Recognition (CVPR94) Seattle, June 1994. (KLT)

- D. Chetverikov, J. Verestoy Tracking feature points: A new Algorithm. In Proc. International Conf. On Pattern Recognition, pp. 1436-1438, 1998.

- David G. Lowe Object Recognition from Local Scale-Invariant Features. In Proc. of the International Conf. On Computer Vision, Corfu, September 1999.

- M. Werman, D. Weinshall Similarity and Affine Invariant Distances Between 2D Point Sets. IEEE Trans. On Pattern Analysis and Machine Int., 17(8):810-814, 1995.

- V. Rvatchev R-functions – Theory and Applications. Kiev, Ukraine, State Publishing House, 1982.

- A. Karkishchenko, S. Butenkov, J. Jimenez Analytical Approach in Pattern Recognition for Parallel Systems. In Proc. International Conference on Intellectual Multiprocessor Systems (IMS99), Taganrog, Russia, 1-5 September 1999.

- A. Karkishchenko, S. Butenkov, O. Semery Analytical Parameterized Models in Computer Vision. In Proc. 6th International Conference on Control, Robotics and Vision (ICARCV 2000), Singapore, 5-8 December 2000

- A. Dold Lectures on algebraic topology. Berlin, Heidelberg, New York: Springer, 1972.

- V. Shapiro. Theory of R-functions and Applications: A Primer, Tech. Report TR 91-1219, Computer Science Department, Cornell University, Ithaca, NY, 1991.